SECTION 02200

SITE PREPARATION

PART 1 - GENERAL

1.1	description

1.	Work Included:  Work shall be completed as described in this section and shall include the furnishing of all labor, material, equipment, and appurtenances to remove all tree stumps, trees, limbs, sod, topsoil, and rubbish from construction area and dispose of said material in an approved location.  Stockpile topsoil in an approved area for later use during final grading and restoration.  The CONTRACTOR shall furnish all labor materials, supplies, and equipment necessary to provide erosion and sediment control during construction of the facilities.

2.	Related Sections:  Additional Sections of the Documents which are referenced in this Section include:

1)	Section 02700 – Bases, Ballasts, Pavements, and Appurtenances

2)	Section 02921 - Seeding

1.2	references

1.	General:  The work shall comply with the most recent standards or tentative standards as published at the date of the contract and as listed in this specification using the abbreviation shown.

2.	Virginia Department of Transportation - Road and Bridge Specifications (VDOT):

1)	106	Control of Material

1.3	DEFINITIONS

1.	Clearing:  Clearing shall consist of the felling, trimming, and cutting of trees into sections and the satisfactory disposal of the trees and other vegetation designated for removal, including down timber, snags, brush, and rubbish occurring in the areas to be cleared.

2.	Grubbing:  Grubbing shall consist of the removal and disposal of brush, stumps, roots larger than 3 inches in diameter, and matted roots from the designated grubbing areas.

3.	Usable Topsoil:  Topsoil to be stockpiled for restoration shall consist of friable clay loam, free from roots, stones, and other undesirable material and shall be capable of supporting a good growth of grass.

4.	Large Trees:  Trees, limbs, and other timber having a diameter of 3 inches and greater shall be disposed of as saw logs, firewood, and other usable material.

5.	Brush:  Brush and tree tops may be chipped, stockpiled, and used for mulch on the project.

1.4	QUALITY ASSURANCE

1.	Coordination:  Coordinate clearing operations with OWNER to limit clearing of work areas.

1.5	PROJECT CONDITIONS

1.	Site Protection:

1)	CONTRACTOR shall provide protection as necessary to prevent damage to existing site improvements or vegetation to remain in place as indicated on the plans.

2)	CONTRACTOR shall protect improvements on adjoining properties and on OWNER's property.

3)	CONTRACTOR shall restore damaged improvements to their original condition, as acceptable to parties having jurisdiction.

PART 2 - products

2.1	Erosion and sediment control

1.	General:  All perimeter erosion control measures shall be installed prior to site disturbance.

PART 3 - execution

3.1	CLEARING AND GRUBBING

1.	Clearing:  All trees, stumps, roots, brush, and other vegetation in the area shall be removed to a depth of 12 inches below finished grades, foundations or slabs unless otherwise indicated on the plans.  All brush, small trees, limbs, sod or rubbish shall be removed from the site.

2.	Grubbing:  Material to be grubbed, together with logs and other organic debris not suitable for foundation purposes, shall be removed to a depth of not less than 12 inches below the original surface level of the ground in areas indicated to be grubbed and in areas indicated as construction areas under this Contract, such as areas buildings, and areas to be paved.  Depressions made by grubbing shall be filled with suitable material and compacted to make the surface conform with the original adjacent surface of the ground.

3.	Staking:  Areas to be cleared shall be staked on the ground by the CONTRACTOR and approved by the OWNER before clearing operations are begun.  CONTRACTOR shall obtain services of a certified Engineer or Land Surveyor for purposes of facilities layout.  Locations to be field staked by the CONTRACTOR and approved by OWNER’s Representative prior to any construction.

3.2	DISPOSAL OF MATERIALS

1.	Disposal:  Carry out disposal of debris and unsuitable or surplus material in accordance with the VDOT 106.  In all cases, the CONTRACTOR will be responsible for obtaining a suitable disposal site.

2.	Burning or Removing from Site:  Logs, stumps, roots, brush, rotten wood, and other refuse from the clearing and grubbing operations, except for salable timber, shall be removed from the site by the CONTRACTOR and shall be disposed of legally off-site.

3.3	Drainage

1.	General:  The CONTRACTOR shall be responsible for proper drainage of the site during construction of the project.  Water shall not be allowed to accumulate in any of the excavated areas.  Storm or ground water collecting on the site during construction shall be removed by pumping, ditching or other suitable means.

3.4	STOCKPILING

1.	Topsoil:  Topsoil shall be stripped from all excavation and fill areas and stockpiled in an approved area until needed for finish grading.  Stockpiles shall be seeded within 10 days of construction.  Silt fence shall also be maintained around the stockpiles until mature vegetation is established on the stockpile.

2.	Spreading of Topsoil:  The spreading of topsoil shall be in accordance with Section 02921 - Seeding in these specifications.

END OF SECTION